# Getting Familiar with Arduino Nano 33, Raspberry Pi Pico, ArduCAM Camera, TensorFlow Lite

## Arduino Nano 33 and TensorFlow Lite

### Basics of TensorFlow

NOC:Practical Machine Learning with TensorFlow

https://nptel.ac.in/courses/106/106/106106213/#

### Predicting a Sine Value

- From the example, google-colab tflite_sinewave_training.ipynb, I generated the sine_model.h and wrote a Arduino sketch tensorlite_ex1.ino.

- I configured and tried to run the example Tensor Lite program, but got this message “Didn't find op for builtin opcode 'FULLY_CONNECTED' version '9'”. I installed the Tensor Lite version 1.0.15-Alpha, which was mentioned in the example.

- I started debugging and reading about the error.

- I used the Arduino example, which is the same as we are doing it. “hello_world.ino” is similar to tensorlite_ex1.ino but it has its pre-generated model header file. It was executed without any errors.

- The values seen on serial port and serial plotter are seems to be a sinusoidal waveform. Which is correct.

- Then, I fixed that the data being generated from our python is somehow not correct.

- Then found that somehow, some parameters are to be changed as below:

##converter.optimizations = [tf.lite.Optimize.OPTIMIZE_FOR_SIZE]

#converter.optimizations = [tf.lite.Optimize.DEFAULT]

#tflite_model = converter.convert()

#open(tflite_model_name + '.tflite', 'wb').write(tflite_model)

#This is giving following error while sine_model.h is used in Arduino

#Didn't find op for builtin opcode 'FULLY_CONNECTED' version '9'

# following code resolved the issue

#https://stackoverflow.com/questions/63620419/firebase-local-model-throws-didnt-find-op-for-builtin-opcode-conv-2d-version

converter = tf.lite.TFLiteConverter.from_keras_model(model)

## ArduCAM Camera

- ArduCAM-M-2MP Camera Shield was connected with Arduino Nano 33. The Camera was not detected, I debugged, but could not resolve.

| ArduCAM | Arduino |
| --- | --- |
| CS | D10 |
| MOSI | D11 |
| MISO | D12 |
| SCK | D13 |
| GND | GND |
| VCC | +5V |
| SDA | A4 |
| SCL | A5 |

- Also programmed the “ArduCAM_Mini_2MP_OV2640_functions.ino” and tried to interact with the camera using “ArduCAM_Host_V2.0_Windows.exe” application. It was giving SPI error.

- Then I connected VCC of the Camera to the 3V3 of the Arduino and it works now.

- I used “ArduCAM_Mini_2MP_OV2640_functions.ino” and then took some pictures and videos using “ArduCAM_Host_V2.0_Windows.exe”.

### Person Detection Example

- Programmed “person_detection.ino” and then get the running algorithm.

## Raspberry Pi Pico

- I have spent hours installing and checking the Pi Pico Toolchain to run an example program.

- It was successfully installed and can run from command prompt.

- I can't detect it when plugged in to different laptops and also, I checked with TECO HW lab. There is a beep when I plug in the Pico, but there is no derive popped on the computer.

- Later, I found that, while inserting the USB to the PC, the Pi Pico RESET button should be pressed.

### Setup pico-sdk and pico-examples using the command prompt and successfully run the blink example on RPI-Pico.

D:\rpi-pico\pico-examples> mkdir build

D:\rpi-pico\pico-examples> cd build

D:\rpi-pico\pico-examples\build>cmake -G "NMake Makefiles" ..

PICO_SDK_PATH is D:/rpi-pico/pico-sdk

PICO platform is rp2040.

PICO target board is pico.

Using board configuration from D:/rpi-pico/pico-sdk/src/boards/include/boards/pico.h

TinyUSB available at D:/rpi-pico/pico-sdk/lib/tinyusb/src/portable/raspberrypi/rp2040; adding USB support.

-- Configuring done

-- Generating done

-- Build files have been written to: D:/rpi-pico/pico-examples/build

D:\rpi-pico\pico-examples\build>nmake

…

…

D:\rpi-pico\pico-examples\build>

### Building your own project and implementing using the command prompt and successfully run the project example on RPI-Pico.

D:\rpi-pico>mkdir pico-empty-project

D:\rpi-pico>cd pico-empty-project

D:\rpi-pico\pico-empty-project>mkdir build

D:\rpi-pico\pico-empty-project>mkdir main

D:\rpi-pico\pico-empty-project>create CMakeLists.txt

D:\rpi-pico\pico-empty-project>copy from pico-exampel example_auto_set_url.cmake and pico_sdk_import.cmake

D:\rpi-pico\pico-empty-project>vim

'vim' is not recognized as an internal or external command,

operable program or batch file.

D:\rpi-pico\pico-empty-project>vi

'vi' is not recognized as an internal or external command,

operable program or batch file.

D:\rpi-pico\pico-empty-project>dir

Volume in drive D is Hussain

Volume Serial Number is C43F-8639

Directory of D:\rpi-pico\pico-empty-project

21/10/2021  04:05    <DIR>          .

21/10/2021  04:05    <DIR>          ..

21/10/2021  04:02    <DIR>          build

21/10/2021  04:04               351 CMakeLists.txt

18/10/2021  03:54               301 example_auto_set_url.cmake

21/10/2021  04:06    <DIR>          main

18/10/2021  03:54             2,825 pico_sdk_import.cmake

3 File(s)          3,477 bytes

4 Dir(s)  18,543,718,400 bytes free

D:\rpi-pico\pico-empty-project>cd build

D:\rpi-pico\pico-empty-project\build>cmake -G "NMake Makefiles" ..

Using PICO_SDK_PATH from environment ('d:\rpi-pico\pico-sdk')

PICO_SDK_PATH is D:/rpi-pico/pico-sdk

Defaulting PICO_PLATFORM to rp2040 since not specified.

Defaulting PICO platform compiler to pico_arm_gcc since not specified.

-- Defaulting build type to 'Release' since not specified.

PICO compiler is pico_arm_gcc

-- The C compiler identification is GNU 10.3.1

-- The CXX compiler identification is GNU 10.3.1

-- The ASM compiler identification is GNU

-- Found assembler: C:/Program Files (x86)/GNU Arm Embedded Toolchain/10 2021.07/bin/arm-none-eabi-gcc.exe

Defaulting PICO target board to pico since not specified.

Using board configuration from D:/rpi-pico/pico-sdk/src/boards/include/boards/pico.h

-- Found Python3: C:/Program Files (x86)/Python39-32/python.exe (found version "3.9.5") found components: Interpreter

TinyUSB available at D:/rpi-pico/pico-sdk/lib/tinyusb/src/portable/raspberrypi/rp2040; adding USB support.

-- Configuring done

-- Generating done

-- Build files have been written to: D:/rpi-pico/pico-empty-project/build

D:\rpi-pico\pico-empty-project\build>nmake

Microsoft (R) Program Maintenance Utility Version 14.29.30136.0

Copyright (C) Microsoft Corporation.  All rights reserved.

Scanning dependencies of target bs2_default

[  1%] Building ASM object pico-sdk/src/rp2_common/boot_stage2/CMakeFiles/bs2_default.dir/compile_time_choice.S.obj

[  3%] Linking ASM executable bs2_default.elf

[  3%] Built target bs2_default

[  5%] Generating bs2_default.bin

[  6%] Generating bs2_default_padded_checksummed.S

[  6%] Built target bs2_default_padded_checksummed_asm

[  8%] Creating directories for 'ELF2UF2Build'

[ 10%] No download step for 'ELF2UF2Build'

[ 11%] No update step for 'ELF2UF2Build'

[ 13%] No patch step for 'ELF2UF2Build'

[ 15%] Performing configure step for 'ELF2UF2Build'

-- The C compiler identification is MSVC 19.29.30136.0

-- The CXX compiler identification is MSVC 19.29.30136.0

-- Detecting C compiler ABI info

-- Detecting C compiler ABI info - done

-- Check for working C compiler: C:/Program Files (x86)/Microsoft Visual Studio/2019/BuildTools/VC/Tools/MSVC/14.29.30133/bin/Hostx86/x86/cl.exe - skipped

-- Detecting C compile features

-- Detecting C compile features - done

-- Detecting CXX compiler ABI info

-- Detecting CXX compiler ABI info - done

-- Check for working CXX compiler: C:/Program Files (x86)/Microsoft Visual Studio/2019/BuildTools/VC/Tools/MSVC/14.29.30133/bin/Hostx86/x86/cl.exe - skipped

-- Detecting CXX compile features

-- Detecting CXX compile features - done

-- Configuring done

-- Generating done

-- Build files have been written to: D:/rpi-pico/pico-empty-project/build/elf2uf2

[ 16%] Performing build step for 'ELF2UF2Build'

Microsoft (R) Program Maintenance Utility Version 14.29.30136.0

Copyright (C) Microsoft Corporation.  All rights reserved.

[ 50%] Building CXX object CMakeFiles/elf2uf2.dir/main.cpp.obj

main.cpp

D:\rpi-pico\pico-sdk\tools\elf2uf2\main.cpp(328): warning C4996: 'fopen': This function or variable may be unsafe. Consider using fopen_s instead. To disable deprecation, use _CRT_SECURE_NO_WARNINGS. See online help for details.

D:\rpi-pico\pico-sdk\tools\elf2uf2\main.cpp(334): warning C4996: 'fopen': This function or variable may be unsafe. Consider using fopen_s instead. To disable deprecation, use _CRT_SECURE_NO_WARNINGS. See online help for details.

[100%] Linking CXX executable elf2uf2.exe

[100%] Built target elf2uf2

[ 18%] No install step for 'ELF2UF2Build'

[ 20%] Completed 'ELF2UF2Build'

[ 20%] Built target ELF2UF2Build

Scanning dependencies of target main

[ 22%] Building C object main/CMakeFiles/main.dir/main.c.obj

[ 23%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_stdlib/stdlib.c.obj

[ 25%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_gpio/gpio.c.obj

[ 27%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_claim/claim.c.obj

[ 28%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_platform/platform.c.obj

[ 30%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_sync/sync.c.obj

[ 32%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_uart/uart.c.obj

[ 33%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_divider/divider.S.obj

[ 35%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_time/time.c.obj

[ 37%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_time/timeout_helper.c.obj

[ 38%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_timer/timer.c.obj

[ 40%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_sync/sem.c.obj

[ 42%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_sync/lock_core.c.obj

[ 44%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_sync/mutex.c.obj

[ 45%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_sync/critical_section.c.obj

[ 47%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_util/datetime.c.obj

[ 49%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_util/pheap.c.obj

[ 50%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/common/pico_util/queue.c.obj

[ 52%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_runtime/runtime.c.obj

[ 54%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_clocks/clocks.c.obj

[ 55%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_irq/irq.c.obj

[ 57%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_irq/irq_handler_chain.S.obj

[ 59%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_pll/pll.c.obj

[ 61%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_vreg/vreg.c.obj

[ 62%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_watchdog/watchdog.c.obj

[ 64%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/hardware_xosc/xosc.c.obj

[ 66%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_printf/printf.c.obj

[ 67%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_bit_ops/bit_ops_aeabi.S.obj

[ 69%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_bootrom/bootrom.c.obj

[ 71%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_divider/divider.S.obj

[ 72%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_double/double_aeabi.S.obj

[ 74%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_double/double_init_rom.c.obj

[ 76%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_double/double_math.c.obj

[ 77%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_double/double_v1_rom_shim.S.obj

[ 79%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_int64_ops/pico_int64_ops_aeabi.S.obj

[ 81%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_float/float_aeabi.S.obj

[ 83%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_float/float_init_rom.c.obj

[ 84%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_float/float_math.c.obj

[ 86%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_float/float_v1_rom_shim.S.obj

[ 88%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_malloc/pico_malloc.c.obj

[ 89%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_mem_ops/mem_ops_aeabi.S.obj

[ 91%] Building ASM object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_standard_link/crt0.S.obj

[ 93%] Building CXX object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_standard_link/new_delete.cpp.obj

[ 94%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_standard_link/binary_info.c.obj

[ 96%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_stdio/stdio.c.obj

[ 98%] Building C object main/CMakeFiles/main.dir/D_/rpi-pico/pico-sdk/src/rp2_common/pico_stdio_uart/stdio_uart.c.obj

[100%] Linking CXX executable main.elf

[100%] Built target main

D:\rpi-pico\pico-empty-project\build>

### Setup pico-sdk and pico-examples using the Visual Studio Code and successfully run the blink example on RPI-Pico.

- Copied fresh copy of pico-example into pico-exmaple1

- created a build directory inside pico-examples1

- started visual studio code from VS2019 Developer command prompt

- Cmake: Configure Environment

Item: PICO_SDK_PATH Value: D:\rpi-pico\pico-sdk

Cmake: Generator

NMake Makefiles

Cmake: Cmake Path

cmake

- open the folder pico-example1

- VS Code will automatically configure the project.

- after that click "build" button at the bottom

- The VSC bottom looks like:

- When the folder is first opened

[variant] Loaded new set of variants

[kit] Successfully loaded 6 kits from C:\Users\Sajjad\AppData\Local\CMakeTools\cmake-tools-kits.json

[proc] Executing command: "C:\Program Files (x86)\GNU Arm Embedded Toolchain\10 2021.07\bin\arm-none-eabi-gcc.exe" -v

[main] Configuring folder: pico-examples2

[proc] Executing command: "C:\Program Files\CMake\bin\cmake.EXE" --no-warn-unused-cli -DCMAKE_EXPORT_COMPILE_COMMANDS:BOOL=TRUE -DCMAKE_BUILD_TYPE:STRING=Debug "-DCMAKE_C_COMPILER:FILEPATH=C:\Program Files (x86)\GNU Arm Embedded Toolchain\10 2021.07\bin\arm-none-eabi-gcc.exe" "-DCMAKE_CXX_COMPILER:FILEPATH=C:\Program Files (x86)\GNU Arm Embedded Toolchain\10 2021.07\bin\arm-none-eabi-g++.exe" -Hd:/rpi-pico/pico-examples2 -Bd:/rpi-pico/pico-examples2/build -G "NMake Makefiles"

[cmake] Not searching for unused variables given on the command line.

[cmake] Using PICO_SDK_PATH from environment ('D:\rpi-pico\pico-sdk')

[cmake] PICO_SDK_PATH is D:/rpi-pico/pico-sdk

[cmake] Defaulting PICO_PLATFORM to rp2040 since not specified.

[cmake] Defaulting PICO platform compiler to pico_arm_gcc since not specified.

[cmake] PICO compiler is pico_arm_gcc

[cmake] -- The C compiler identification is GNU 10.3.1

[cmake] -- The CXX compiler identification is GNU 10.3.1

[cmake] -- The ASM compiler identification is GNU

[cmake] -- Found assembler: C:/Program Files (x86)/GNU Arm Embedded Toolchain/10 2021.07/bin/arm-none-eabi-gcc.exe

[cmake] Using regular optimized debug build (set PICO_DEOPTIMIZED_DEBUG=1 to de-optimize)

[cmake] Defaulting PICO target board to pico since not specified.

[cmake] Using board configuration from D:/rpi-pico/pico-sdk/src/boards/include/boards/pico.h

[cmake] -- Found Python3: C:/Program Files (x86)/Python39-32/python.exe (found version "3.9.5") found components: Interpreter

[cmake] TinyUSB available at D:/rpi-pico/pico-sdk/lib/tinyusb/src/portable/raspberrypi/rp2040; adding USB support.

[cmake] Compiling TinyUSB with CFG_TUSB_DEBUG=1

[cmake] -- Configuring done

[cmake] -- Generating done

[cmake] -- Build files have been written to: D:/rpi-pico/pico-examples2/build

- When the “build” button at the bottom bar is pressed

[main] Building folder: pico-examples1

…

…

[build] [100%] Built target hello_watchdog

[build] Build finished with exit code 0

### Building your own project and implementing using the Visual Studio Code and successfully run the project example on RPI-Pico.

- create a folder called myblinkvsc

- create your main.c file

- create CMakeLists file

- started visual studio code from VS2019 Developer command prompt

- open the folder myblinkvsc

- VS Code will automatically configure the project.

- after that click "build" button at the bottom

- There was an error while building the projects, then after a through google surfing found that the an older CMake version should be used. There I uninstalled the latest version and installed CMake 3.20.6. (AR10B2~1.EXE: error: ´╗┐CMakeFiles/hello_serial.dir/hello_serial.c.obj: No such file or directory)